Development of Pre-clinical System for Shaping Plasticity via Reflex Conditioning paired with Vagus Nerve Stimulation

Addressing Feedback

Dr. Eric Meyers:

Specific Aims: Development of Pre-clinical System for Shaping Plasticity via Reflex Conditioning paired with Vagus Nerve Stimulation

Interventions that foster neural plasticity in spared sensorimotor networks have long been recognized for their potential to restore functional after neurological injury. However, meaningful recovery form severe injuries necessitates an extensive and precise reorganization of neural connections [cite]. While various methods have been proposed to enhance synaptic plasticity to facilitate this “rewiring,” many have struggled to transition to successful clinical applications. These historical failures are often attributed to the significant challenge of precisely targeting specific, task-relevant synapses and lack of testing in models that reflect clinical environments. [Need to cite this whole section.

Vagus nerve stimulation (VNS) has surfaced as a potent neuromodulatory strategy to accelerate recovery following conditions like stroke or spinal cord injury. VNS works by triggering the widespread, phasic release of key neuromodulators—specifically acetylcholine, norepinephrine, and serotonin—which collectively prime the central nervous system environment for plasticity. While this global release can facilitate long-lasting improvements in motor function, VNS therapy may be limited when administered without a way to focus these effects, hinting at a lack of specificity of VNS therapy alone. This is because VNS-directed plasticity utilizes "functional targeting" to reinforce only the specific neural networks active during stimulation. Since global neuromodulator release alone lacks precision, VNS must be paired with targeted, circuit-oriented interventions to address the maladaptive hyperreflexia that causes spasticity. This combination allows VNS to effectively treat the underlying neural causes of specific motor dysfunctions. [Need to cite this whole section]

This project aims to address that gap by supplementing a VNS-based therapy paradigm with an intervention known as H-Reflex Operant Conditioning (HROC), a targeted therapy known to drive plasticity to the spinal cord, to form a new therapy, VNS-enhanced H-reflex Operant Conditioing (VeHROC). This paradigm will necessitate the development of a pre-clinical system capable of capturing electrophysiological locomotor data from the lower limb muscles and delivering precisely timed stimulations of VNS to enhance the therapeutic effects of HROC. Both effects of HROC and implanted VNS will be evaluated.

By comparing HROC and VeHROC with identical investigative pipelines, I aim to determine whether this combination/addition of VNS with HROC will augment spinal conditioning effects and lead to superior locomotor recovery compared to either therapy utilized alone.

This research will focus on developing multi-modal pre-clinical system capable of real-time recording and deliver VNS and evoke the Hoffman Reflex (H-Reflex) to study the progression of locomotor recovery with electrophysiology metrics such as electromyography (EMG), electroencephalography (EEG), and diaphragm electromyography (dEMG). Ultimately, these efforts support the advancement of rehabilitative therapies that address the underlying spinal circuits of motor disorders and advancing translation toward clinical therapies targeted at driving plasticity to the spinal cord.

Impact Statement:

# Aim 1: Develop and Validate Pre-Clinical Technology for Real-Time Recording and Stimulation

Build a closed-loop lab-based system that can simultaneously record biological signals and deliver precisely timed stimulation in response to designed behavioral task paradigms. This system will acquire EMG to measure muscle activity in multiple anatomical locations (leg, jaw, diaphragm), as well as EEG to monitor brain activity and network synchrony. This system will also be equipped to deliver stimulation to different nerves, namely the tibial nerve to evoke the H-Reflex (a measure of spinal reflex excitability) and the left vagal nerve for Vagus Nerve Stimulation (VNS). This system will include an Open Ephys Acquisition Board for acquiring data, as well as programmable 4100 AM Stimulators for the main pieces of instrumentation. Additional components will include a commutator to allow communication between the instrumentation and the rat cage/booth. Recording hardware will include custom built multi-channel headcaps and recording headstages, recording electrodes (EMG and EEG), and lastly active components will include nerve stimulating cuffs (VNS Cuffs and Tibial Cuffs).

The system will be built alongside an in-house developed python software, The H-Reflex App, that will interface mainly with the Open Ephys GUI & Open Ephys Acquisition Board, as well as the programmable 4100 AM Stimulator(s). The H-Reflex App will allow for autonomous execution of experiments including recording electrophysiological activity, eliciting stimulation events, and delivering reward stimulations on successful trails when necessary. The system will be configured with TTL sync lines to enable time-synchronized recording and real-time event triggering. This system will serve as the multi-faceted platform for experimental-behavioral paradigms to support all aspects of the project.

Aim 2: Observe Long-term Electrophysiological Effects of H-Reflex Conditioning in Awake Rats

Evaluate electrophysiological signals and method of collection

Aim 3: Identify Reliable Electrophysiological Biomarkers of VNS for Closed-Loop VNS Delivery

Compare …

Stages of H-reflex App

S1: Recruitment Curve

S2: Control-Mode

|

S3: Down-conditioning with Pellet

S4: Up-conditioning with Pellet

S5: Down-conditioning with VNS

S6: Up-conditioning with VNS

Trial per-Trial for Conditioning

Run S2: (To get historical data and for setting initial threshold)

Run S3/S4/S5/S6: Depending on goal

S3-6 will have a moving window of 100 trials (1 in, 1 out), and everything recalculate the distribution and threshold for the 30th percentile

APPROACH

# Aim 1: ………….

Methods

Pre-Clinical Studies

Study Design and Recordings

Cohort

Data Analysis

Expected Results and Interpretations

Potential Pitfalls, Contingencies, and Alternative Approaches

Summary of Aim 1

Aim 2: …..

Methods

Pre-Clinical Studies

Study Design

Participants

Data Analysis

Expected Results and Interpretations

Potential Pitfalls, Contingencies, and Alternative Approaches

Summary of Aim 2

Aim 3: …….

Methods

Pre-Clinical Studies

Study Design

Participants

Data Analysis

Expected Results and Interpretations

Potential Pitfalls, Contingencies, and Alternative Approaches

Summary of Aim 3

Apparatus and Procedures

IACUC Standards and Guidelines

Rationale and Significance

The Need for Biomarker-Guided Rehabilitation

Spinal cord injury and stroke are …

Importance of Real-Time Multimodal Platforms

Vagus nerve stimulation (VNS) …

Relevance of Comparing Implanted VNS and tAN

Longitudinal Biomarkers as Predictors of Recovery

Broader Impact and Future Applications

## Project Timeline

I expect the remaining proposed aims will take two years to complete.

| Task | Year 1 | Year 2 | Year 3 | Year 4 |
| --- | --- | --- | --- | --- |
| Aim 1 |  |  |  |  |
| Aim 2 |  |  |  |  |
| Aim 3 |  |  |  |  |
| Manuscript Prep |  |  |  |  |

[1]. Meyers EC, Solorzano BR, James J, Ganzer PD, Lai ES, Rennaker RL, et al. Vagus Nerve Stimulation Enhances Stable Plasticity and Generalization of Stroke Recovery. Stroke 2018:10.1161/STROKEAHA.117.019202.

[2]. Ganzer PD, Darrow MJ, Meyers EC, Solorzano BR, Ruiz AD, Robertson NM, et al. Closed-loop Neuroprosthesis Restores Network Connectivity and Motor 2 Control after Spinal Cord Injury. Elife n.d

[3] Ganzer et al.,Closed-loop neuromodulation restores network connectivity and motor control after spinal cord injury, 2018)

[4]. Dawson J, Pierce D, Dixit A, Kimberley TJ, Robertson M, Tarver B, et al. Safety, Feasibility, and Efficacy of Vagus Nerve Stimulation Paired With Upper-Limb Rehabilitation After Ischemic Stroke. Stroke 2015;47:143–50. 10.1161/STROKEAHA.115.010477. [PubMed: 26645257]

Citations (From Austin’s Document):

[1]	N. D. Engineer et al., “Reversing pathological neural activity using targeted plasticity,” Nature, vol. 470, no. 7332, pp. 101–104, Feb. 2011, doi: 10.1038/nature09656.

[2]	J. Dawson et al., “Safety, feasibility, and efficacy of vagus nerve stimulation paired with upper-limb rehabilitation after ischemic stroke,” Stroke, vol. 47, no. 1, pp. 143–150, 2016.

[3]	M. P. Kilgard et al., “Closed-loop vagus nerve stimulation aids recovery from spinal cord injury,” Nature, pp. 1–7, May 2025, doi: 10.1038/s41586-025-09028-5.

[4]	J. Dawson et al., “Vagus nerve stimulation paired with rehabilitation for upper limb motor function after ischaemic stroke (VNS-REHAB): a randomised, blinded, pivotal, device trial,” The Lancet, vol. 397, no. 10284, pp. 1545–1553, Apr. 2021, doi: 10.1016/S0140-6736(21)00475-X.

[5]	J. Dawson et al., “Vagus nerve stimulation paired with rehabilitation for upper limb motor impairment and function after chronic ischemic stroke: subgroup analysis of the randomized, blinded, pivotal, VNS-REHAB device trial,” Neurorehabil. Neural Repair, vol. 37, no. 6, pp. 367–373, 2023.

[6]	E. J. Hammond, B. M. Uthman, S. A. Reid, and B. J. Wilder, “Electrophysiological Studies of Cervical Vagus Nerve Stimulation in Humans: I. EEG Effects,” Epilepsia, vol. 33, no. 6, pp. 1013–1020, 1992, doi: 10.1111/j.1528-1157.1992.tb01752.x.

[7]	M. Ravan, S. Sabesan, and O. D’Cruz, “On Quantitative Biomarkers of VNS Therapy Using EEG and ECG Signals,” IEEE Trans. Biomed. Eng., vol. 64, no. 2, pp. 419–428, Feb. 2017, doi: 10.1109/TBME.2016.2554559.

[8]	V. Sivaji et al., “ReStore: A wireless peripheral nerve stimulation system,” J. Neurosci. Methods, vol. 320, pp. 26–36, 2019.

[9]	J. Y. Y. Yap, C. Keatch, E. Lambert, W. Woods, P. R. Stoddart, and T. Kameneva, “Critical Review of Transcutaneous Vagus Nerve Stimulation: Challenges for Translation to Clinical Practice,” Front. Neurosci., vol. 14, p. 284, Apr. 2020, doi: 10.3389/fnins.2020.00284.

[10]	B. W. Badran and C. W. Austelle, “The Future Is Noninvasive: A Brief Review of the Evolution and Clinical Utility of Vagus Nerve Stimulation,” Focus J. Life Long Learn. Psychiatry, vol. 20, no. 1, pp. 3–7, Jan. 2022, doi: 10.1176/appi.focus.20210023.

[11]	I. G. Gurtubay et al., “Immediate effects and duration of a short and single application of transcutaneous auricular vagus nerve stimulation on P300 event related potential,” Front. Neurosci., vol. 17, Mar. 2023, doi: 10.3389/fnins.2023.1096865.

[12]	A. D. Farmer et al., “International Consensus Based Review and Recommendations for Minimum Reporting Standards in Research on Transcutaneous Vagus Nerve Stimulation (Version 2020),” Front. Hum. Neurosci., vol. 14, p. 568051, 2020, doi: 10.3389/fnhum.2020.568051.

[13]	S. S. Baig et al., “tVNS in Stroke: A Narrative Review on the Current State and the Future,” Stroke, vol. 54, no. 10, pp. 2676–2687, Oct. 2023, doi: 10.1161/STROKEAHA.123.043414.

[14]	W. He et al., “Auricular Acupuncture and Vagal Regulation,” Evid.-Based Complement. Altern. Med. ECAM, vol. 2012, p. 786839, 2012, doi: 10.1155/2012/786839.

[15]	E. T. Peuker and T. J. Filler, “The nerve supply of the human auricle,” Clin. Anat. N. Y. N, vol. 15, no. 1, pp. 35–37, Jan. 2002, doi: 10.1002/ca.1089.

[16]	S. Vanneste, J. Martin, R. L. Rennaker, and M. P. Kilgard, “Pairing sound with vagus nerve stimulation modulates cortical synchrony and phase coherence in tinnitus: An exploratory retrospective study,” Sci. Rep., vol. 7, no. 1, p. 17345, Dec. 2017, doi: 10.1038/s41598-017-17750-y.

[17]	E. G. Morrison et al., “Characterization of Vagus Nerve Stimulation (VNS) Dose-Dependent Effects on EEG Power Spectrum and Synchronization,” Biomedicines, vol. 12, no. 3, p. 557, Mar. 2024, doi: 10.3390/biomedicines12030557.

[18]	F. Bartolomei et al., “How does vagal nerve stimulation (VNS) change EEG brain functional connectivity?,” Epilepsy Res., vol. 126, pp. 141–146, Oct. 2016, doi: 10.1016/j.eplepsyres.2016.06.008.

[19]	R. L. Verrier, B. D. Nearing, B. Olin, P. Boon, and S. C. Schachter, “Baseline elevation and reduction in cardiac electrical instability assessed by quantitative T-wave alternans in patients with drug-resistant epilepsy treated with vagus nerve stimulation in the AspireSR E-36 trial,” Epilepsy Behav., vol. 62, pp. 85–89, Sept. 2016, doi: 10.1016/j.yebeh.2016.06.016.

[20]	R. Agarwal, E. Mokelke, S. B. Ruble, and C. M. Stolen, “Vagal Nerve Stimulation Evoked Heart Rate Changes and Protection from Cardiac Remodeling,” J Cardiovasc. Transl. Res., vol. 9, no. 1, pp. 67–76, Feb. 2016, doi: 10.1007/s12265-015-9668-7.

[21]	A. Genç, F. E. Uçan Tokuç, and M. Korucuk, “Effects of vagal nerve stimulation parameters on heart rate variability in epilepsy patients,” Front. Neurol., vol. 15, p. 1490887, Oct. 2024, doi: 10.3389/fneur.2024.1490887.

[22]	C. D. Guerrero-Mendez and A. F. Ruiz-Olaya, “Coherence-based connectivity analysis of EEG and EMG signals during reach-to-grasp movement involving two weights,” Brain-Comput. Interfaces, vol. 9, no. 3, pp. 140–154, July 2022, doi: 10.1080/2326263X.2022.2029308.

[23]	G. Konakoğlu, A. V. Özden, H. Solmaz, and C. Bildik, “The effect of auricular vagus nerve stimulation on electroencephalography and electromyography measurements in healthy persons,” Front. Physiol., vol. 14, July 2023, doi: 10.3389/fphys.2023.1215757.

[24]	E. Udina, J. D’Amico, A. J. Bergquist, and M. A. Gorassini, “Amphetamine increases persistent inward currents in human motoneurons estimated from paired motor-unit activity,” J. Neurophysiol., vol. 103, no. 3, pp. 1295–1303, Mar. 2010, doi: 10.1152/jn.00734.2009.

[25]	O. Sharon, F. Fahoum, and Y. Nir, “Transcutaneous Vagus Nerve Stimulation in Humans Induces Pupil Dilation and Attenuates Alpha Oscillations,” J. Neurosci. Off. J. Soc. Neurosci., vol. 41, no. 2, pp. 320–330, Jan. 2021, doi: 10.1523/JNEUROSCI.1361-20.2020.

[26]	B. Lloyd, F. Wurm, R. De Kleijn, and S. Nieuwenhuis, “Short-term transcutaneous vagus nerve stimulation increases pupil size but does not affect EEG alpha power: A replication of Sharon et al. (2021, Journal of Neuroscience),” Brain Stimulat., vol. 16, no. 4, pp. 1001–1008, July 2023, doi: 10.1016/j.brs.2023.06.010.

[27]	J. E. Bucksot, K. M. Castelan, S. K. Skipton, and S. A. Hays, “Parametric characterization of the rat Hering-Breuer reflex evoked with implanted and non-invasive vagus nerve stimulation,” Exp. Neurol., vol. 327, p. 113220, 2020.

[28]	S. M. MacDonald, C. Tin, G. Song, and C. Poon, “Use‐dependent learning and memory of the Hering–Breuer inflation reflex in rats,” Exp. Physiol., vol. 94, no. 2, pp. 269–278, 2009.

[29]	H. Wang, H. Zhang, G. Song, and C.-S. Poon, “Modulation of Hering-Breuer reflex by ventrolateral pons,” Adv. Exp. Med. Biol., vol. 605, pp. 387–392, 2008, doi: 10.1007/978-0-387-73693-8_68.

[30]	Y. Nagai, C. I. Jones, and A. Sen, “Galvanic skin response (GSR)/electrodermal/skin conductance biofeedback on epilepsy: a systematic review and meta-analysis,” Front. Neurol., vol. 10, p. 377, 2019.

[31]	J. Polich, “Updating P300: An Integrative Theory of P3a and P3b,” Clin. Neurophysiol. Off. J. Int. Fed. Clin. Neurophysiol., vol. 118, no. 10, p. 2128, June 2007, doi: 10.1016/j.clinph.2007.04.019.

[32]	A. H. Neuhaus et al., “P300 is enhanced in responders to vagus nerve stimulation for treatment of major depressive disorder,” J. Affect. Disord., vol. 100, no. 1, pp. 123–128, June 2007, doi: 10.1016/j.jad.2006.10.005.

[33]	L. De Taeye et al., “The P3 Event-Related Potential is a Biomarker for the Efficacy of Vagus Nerve Stimulation in Patients with Epilepsy,” Neurotherapeutics, vol. 11, no. 3, pp. 612–622, July 2014, doi: 10.1007/s13311-014-0272-3.

[34]	J. L. Chang et al., “Transcutaneous Auricular Vagus Nerve Stimulation (tAVNS) Delivered During Upper Limb Interactive Robotic Training Demonstrates Novel Antagonist Control for Reaching Movements Following Stroke,” Front. Neurosci., vol. 15, Nov. 2021, doi: 10.3389/fnins.2021.767302.

[35]	J. D. Epperson et al., “Characterization of an algorithm for autonomous, closed-loop neuromodulation during motor rehabilitation,” Neurorehabil. Neural Repair, vol. 38, no. 7, pp. 493–505, 2024.

[36]	J. Epperson, E. Meyers, D. Pruitt, J. Wright, M. Kilgard, and S. Hays, “Characterization of an algorithm for autonomous, closed-loop neuromodulation during stroke rehabilitation,” Arch. Phys. Med. Rehabil., vol. 102, no. 10, p. e98, 2021.

[37]	B. A. Porter et al., “Repeatedly pairing vagus nerve stimulation with a movement reorganizes primary motor cortex,” Cereb. Cortex, vol. 22, no. 10, pp. 2365–2374, 2012.

[38]	J. Lanzone et al., “Vagal nerve stimulation cycles alter EEG connectivity in drug-resistant epileptic patients: A study with graph theory metrics,” Clin. Neurophysiol., vol. 142, pp. 59–67, Oct. 2022, doi: 10.1016/j.clinph.2022.07.503.

[39]	Y. Ding, K. Guo, X. Wang, M. Chen, X. Li, and Y. Wu, “Brain functional connectivity and network characteristics changes after vagus nerve stimulation in patients with refractory epilepsy,” Transl. Neurosci., vol. 14, no. 1, p. 20220308, Jan. 2023, doi: 10.1515/tnsci-2022-0308.

[40]	T. Cheng et al., “A predictive model combining connectomics and entropy biomarkers to discriminate long‐term vagus nerve stimulation efficacy for pediatric patients with drug‐resistant epilepsy,” CNS Neurosci. Ther., vol. 30, no. 7, p. e14751, 2024.

[41]	J. Ma et al., “A prediction model integrating synchronization biomarkers and clinical features to identify responders to vagus nerve stimulation among pediatric patients with drug‐resistant epilepsy,” CNS Neurosci. Ther., vol. 28, no. 11, pp. 1838–1848, 2022.

[42]	C. Bodin et al., “Responders to vagus nerve stimulation (VNS) in refractory epilepsy have reduced interictal cortical synchronicity on scalp EEG,” Epilepsy Res., vol. 113, pp. 98–103, July 2015, doi: 10.1016/j.eplepsyres.2015.03.018.

[43]	S. Vespa et al., “Vagus Nerve Stimulation Elicits Sleep EEG Desynchronization and Network Changes in Responder Patients in Epilepsy,” Neurotherapeutics, vol. 18, no. 4, pp. 2623–2638, Oct. 2021, doi: 10.1007/s13311-021-01124-4.

[44]	V. Danthine et al., “Electroencephalogram synchronization measure as a predictive biomarker of Vagus nerve stimulation response in refractory epilepsy: A retrospective study,” PloS One, vol. 19, no. 6, p. e0304115, 2024, doi: 10.1371/journal.pone.0304115.

[45]	E. D. Musselman, I. Raha, N. A. Pelot, and W. M. Grill, “Scaling of vagus nerve stimulation parameters does not achieve equivalent nerve responses across species,” Bioelectron. Med., vol. 11, no. 1, p. 11, May 2025, doi: 10.1186/s42234-025-00174-9.

[46]	E. D. Musselman, N. A. Pelot, and W. M. Grill, “Validated computational models predict vagus nerve stimulation thresholds in preclinical animals and humans,” J. Neural Eng., vol. 20, no. 3, p. 036032, June 2023, doi: 10.1088/1741-2552/acda64.

[47]	S. Avrillon, F. Hug, S. N. Baker, C. Gibbs, and D. Farina, “Tutorial on MUedit: An open-source software for identifying and analysing the discharge timing of motor units from electromyographic signals,” J. Electromyogr. Kinesiol., vol. 77, p. 102886, 2024, doi: 10.1016/j.jelekin.2024.102886.

[48]	K. Kaduk, A. Petrella, S. J. Müller, J. Koenig, and N. B. Kroemer, “Non-Invasive Auricular Vagus Nerve Stimulation Decreases Heart Rate Variability Independent of Caloric Load,” Psychophysiology, vol. 62, no. 2, p. e70017, Feb. 2025, doi: 10.1111/psyp.70017.

[49]	E. Germany Morrison et al., “Characterization of Vagus Nerve Stimulation (VNS) Dose-Dependent Effects on EEG Power Spectrum and Synchronization,” Biomedicines, vol. 12, no. 3, p. 557, Mar. 2024, doi: 10.3390/biomedicines12030557.

[50]	N. D. Engineer et al., “Reversing pathological neural activity using targeted plasticity,” Nature, vol. 470, no. 7332, pp. 101–104, Feb. 2011, doi: 10.1038/nature09656.

[51]	R. Coa et al., “Estimated EEG functional connectivity and aperiodic component induced by vagal nerve stimulation in patients with drug-resistant epilepsy,” Front. Neurol., vol. 13, p. 1030118, 2022, doi: 10.3389/fneur.2022.1030118.

[52]	S. Vespa et al., “Vagus nerve stimulation-induced laryngeal motor evoked potentials: a possible biomarker of effective nerve activation,” Front. Neurosci., vol. 13, p. 880, 2019.

[53]	X. Xi et al., “Construction and analysis of cortical–muscular functional network based on EEG-EMG coherence using wavelet coherence,” Neurocomputing, vol. 438, pp. 248–258, 2021.

[54]	A. C. Schomer, B. D. Nearing, S. C. Schachter, and R. L. Verrier, “Vagus nerve stimulation reduces cardiac electrical instability assessed by quantitative T-wave alternans analysis in patients with drug-resistant focal epilepsy,” Epilepsia, vol. 55, no. 12, pp. 1996–2002, Dec. 2014, doi: 10.1111/epi.12855.

[55]	I. Naggar, K. Nakase, J. Lazar, L. Salciccioli, I. Selesnick, and M. Stewart, “Vagal control of cardiac electrical activity and wall motion during ventricular fibrillation in large animals,” Auton. Neurosci., vol. 183, pp. 12–22, July 2014, doi: 10.1016/j.autneu.2014.01.009.

[56]	C.-S. Poon and G. Song, “Bidirectional plasticity of pontine pneumotaxic postinspiratory drive: implication for a pontomedullary respiratory central pattern generator,” Prog. Brain Res., vol. 209, pp. 235–254, 2014, doi: 10.1016/B978-0-444-63274-6.00012-6.

[57]	Y.-C. Chang et al., “Quantitative estimation of nerve fiber engagement by vagus nerve stimulation using physiological markers,” Brain Stimulat., vol. 13, no. 6, pp. 1617–1630, Nov. 2020, doi: 10.1016/j.brs.2020.09.002.

[58]	A. G. Butler, E. L. O’Callaghan, A. M. Allen, and S. J. McDougall, “Use of a physiological reflex to standardize vagal nerve stimulation intensity improves data reproducibility in a memory extinction assay,” Brain Stimulat., vol. 14, no. 2, pp. 450–459, 2021, doi: 10.1016/j.brs.2021.02.012.

[59]	V. Desbeaumes Jodoin, P. Lespérance, D. K. Nguyen, M.-P. Fournier-Gosselin, and F. Richer, “Effects of vagus nerve stimulation on pupillary function,” Int. J. Psychophysiol., vol. 98, no. 3, Part 1, pp. 455–459, Dec. 2015, doi: 10.1016/j.ijpsycho.2015.10.001.

[60]	Z. Mridha et al., “Graded recruitment of pupil-linked neuromodulation by parametric stimulation of the vagus nerve,” Nat Commun, vol. 12, no. 1, p. 1539, Mar. 2021, doi: 10.1038/s41467-021-21730-2.

[61]	S. Vespa et al., “Characterization of vagus nerve stimulation-induced pupillary responses in epileptic patients,” Brain Stimulat., vol. 15, no. 6, pp. 1498–1507, Nov. 2022, doi: 10.1016/j.brs.2022.11.002.

[62]	A. Torres Sánchez et al., “VNS-induced dose-dependent pupillary response in refractory epilepsy,” Clin. Neurophysiol., vol. 171, pp. 67–75, Mar. 2025, doi: 10.1016/j.clinph.2025.01.006.

[63]	I. J. Kirk, M. J. Spriggs, and R. L. Sumner, “Human EEG and the mechanisms of memory: investigating long-term potentiation (LTP) in sensory-evoked potentials,” J. R. Soc. N. Z., vol. 51, no. 1, pp. 24–40, 2021.

[64]	C. Delcamp, C. Cormier, A. Chalard, D. Amarantini, and D. Gasq, “Changes in intermuscular connectivity during active elbow extension reveal a functional simplification of motor control after stroke,” Front. Neurosci., vol. 16, p. 940907, 2022.

[65]	C. W. Tsao et al., “Heart disease and stroke statistics—2023 update: a report from the American Heart Association,” Circulation, vol. 147, no. 8, pp. e93–e621, 2023.

[66]	National Spinal Cord Injury Statistical Center, “Traumatic Spinal Cord Injury Facts and Figures at a Glance,” University of Alabama at Birmingham, Birmingham, AL, 2022.

[67]	L. A. Ingram, A. A. Butler, M. A. Brodie, S. R. Lord, and S. C. Gandevia, “Quantifying upper limb motor impairment in chronic stroke: a physiological profiling approach,” J. Appl. Physiol., vol. 131, no. 3, pp. 949–965, 2021.

[68]	S. A. Chohan, P. K. Venkatesh, and C. H. How, “Long-term complications of stroke and secondary prevention: an overview for primary care physicians,” Singapore Med. J., vol. 60, no. 12, p. 616, 2019.

[69]	A. Alizadeh, S. M. Dyck, and S. Karimi-Abdolrezaee, “Traumatic spinal cord injury: an overview of pathophysiology, models and acute injury mechanisms,” Front. Neurol., vol. 10, p. 282, 2019.

[70]	R. Stockley, R. Peel, K. Jarvis, and L. Connell, “Current therapy for the upper limb after stroke: a cross-sectional survey of UK therapists,” BMJ Open, vol. 9, no. 9, p. e030262, 2019.

[71]	K. Nas, L. Yazmalar, V. Şah, A. Aydın, and K. Öneş, “Rehabilitation of spinal cord injuries,” World J. Orthop., vol. 6, no. 1, p. 8, 2015.

[72]	P. Langhorne, J. Bernhardt, and G. Kwakkel, “Stroke rehabilitation,” The Lancet, vol. 377, no. 9778, pp. 1693–1702, 2011.

[73]	S. Thuret, L. D. Moon, and F. H. Gage, “Therapeutic interventions after spinal cord injury,” Nat. Rev. Neurosci., vol. 7, no. 8, pp. 628–643, 2006.

[74]	D. De Ridder, S. Vanneste, N. D. Engineer, and M. P. Kilgard, “Safety and efficacy of vagus nerve stimulation paired with tones for the treatment of tinnitus: a case series,” Neuromodulation J. Int. Neuromodulation Soc., vol. 17, no. 2, pp. 170–179, Feb. 2014, doi: 10.1111/ner.12127.

[75]	M. I. Hariz, “Complications of deep brain stimulation surgery,” Mov. Disord. Off. J. Mov. Disord. Soc., vol. 17, no. S3, pp. S162–S166, 2002.

[76]	S. A. Hays, R. L. Rennaker II, and M. P. Kilgard, “How to fail with paired VNS therapy,” Brain Stimulat., vol. 16, no. 5, pp. 1252–1258, 2023.

[77]	N. A. Pelot et al., “Quantified morphology of the cervical and subdiaphragmatic vagus nerves of human, pig, and rat,” Front. Neurosci., vol. 14, p. 601479, 2020.

[78]	C. J. Davis, E. D. Musselman, W. M. Grill, and N. A. Pelot, “Fibers in smaller fascicles have lower activation thresholds with cuff electrodes due to thinner perineurium and smaller cross-sectional area,” J. Neural Eng., vol. 20, no. 2, p. 026032, Apr. 2023, doi: 10.1088/1741-2552/acc42b.

[79]	A. R. Upadhye et al., “Fascicles split or merge every∼ 560 microns within the human cervical vagus nerve,” J. Neural Eng., vol. 19, no. 5, p. 054001, 2022.

[80]	Baylor Research Institute, “Wireless Nerve Stimulation Device To Enhance Recovery After Stroke,” clinicaltrials.gov, 2023. [Online]. Available: https://clinicaltrials.gov/study/NCT04534556

[81]	Baylor Research Institute, “Targeted Plasticity Therapy for Upper Limb Rehabilitation in Spinal Cord Injuries,” clinicaltrials.gov, 2023. [Online]. Available: https://clinicaltrials.gov/study/NCT04288245

[82]	J. Dawson et al., “Vagus nerve stimulation paired with rehabilitation for upper limb motor function after ischaemic stroke (VNS-REHAB): a randomised, blinded, pivotal, device trial,” The Lancet, vol. 397, no. 10284, pp. 1545–1553, Apr. 2021, doi: 10.1016/S0140-6736(21)00475-X.

[83]	W. He et al., “Auricular acupuncture and vagal regulation,” Evidence‐Based Complement. Altern. Med., vol. 2012, no. 1, p. 786839, 2012.